If at any time one of your components gets hot, immediately disconnect your battery, consider the probable cause, and then discuss the issues and corrective action with one of the student instructors.

### 1. DES Soldering Resources

Please familiarize yourself with the resources the DES provides is support of your soldering efforts by reviewing the photo below and identifying the assets by name.

### 2. Miniature (Micro) Toggle Switch Soldering

For your first ‘required’ soldering task you are asked to solder two, solid core, 22 AWG wires to the leads of a miniature toggle switch similar to the one depicted to the right. One wire will be soldered to the center lug and the other wire soldered to an outer lug (it doesn’t matter which one). The remaining outer lug remains free. After soldering, heat shrink tubing is applied to strengthen the joints and to guard against a short circuit. Please watch this video prior to soldering day,

https://www.youtube.com/watch?v=pfbs182mvIc