// PROJECT  : IRWakeUp

// PURPOSE  : Basic Intro to Watchdog Timer IR Triggered Sleep Mode

// COURSE   : TEJ3M

// AUTHOR   : AI and a little tweaking

// DATE     : 2026 05 10

// MCU      : 328p (Nano)

// STATUS   : Working

#include <IRremote.h>     //Library V4.7

#include <avr/sleep.h>

#include <avr/wdt.h>

const int RECV_PIN = 2;   // IR Receiver connected to Pin 2

const unsigned long SLEEP_COMMAND = 0xED126B86;  //POWER ON/OFF ON LCD REMOTE

const int AWAKE_PIN = 12; // Status LED for awake/sleep intervals

void setup() {

Serial.begin(9600);

IrReceiver.begin(RECV_PIN, ENABLE_LED_FEEDBACK);

pinMode(LED_BUILTIN, OUTPUT);

pinMode(AWAKE_PIN, OUTPUT);

digitalWrite(AWAKE_PIN,HIGH);

}

void loop() {

if (IrReceiver.decode()) {

unsigned long command = IrReceiver.decodedIRData.decodedRawData;

Serial.print("Received: ");

Serial.println(command, HEX);

if (command == SLEEP_COMMAND) {

digitalWrite(AWAKE_PIN,LOW);

Serial.println("Going to sleep...");

delay(100); // Allow serial to flush

enterSleep();

}

IrReceiver.resume();

}

}

// Watchdog Interrupt Service Routine (ISR)

ISR(WDT_vect) {

// This is called when the WDT "bites"

// It wakes the MCU and continues execution after sleep_mode()

digitalWrite(AWAKE_PIN,HIGH);

}

void enterSleep() {

set_sleep_mode(SLEEP_MODE_PWR_DOWN); // Deepest sleep

// Configure Watchdog Timer for 8-second interval

cli(); // Disable interrupts

wdt_reset();

WDTCSR |= (1 << WDCE) | (1 << WDE);

// Set interrupt mode (WDIE) and 8s timeout (WDP3 and WDP0)

WDTCSR = (1 << WDIE) | (1 << WDP3) | (1 << WDP0);

sei(); // Enable interrupts

sleep_enable();

digitalWrite(LED_BUILTIN, LOW); // Turn off LED to save power

sleep_mode();                   // CPU stops here until WDT or IR interrupt

// --- Wakes up here after WDT timeout ---

sleep_disable();

wdt_disable(); // Stop WDT after waking

Serial.println("Woke up!");

}