// PROJECT  :BitsandBytes

// PURPOSE  :Preparation for Register-Level Coding

// DEVICE   :*

// AUTHOR   :C. D'Arcy

// DATE     :*

// MCU      :*

// COURSE   :ICSnU-E

// STATUS   :Working

char buff[60];  //large enough buffer to handle all output strings

void setup() {

Serial.begin(9600);

while (!Serial);

//define an 8 bit value (reg is short for register)

uint8_t reg = 0x07;  // 7(16) = 7(10) = B00000111

// without leading zeroes...

Serial.println("The value " + String(reg) + " in binary is " + String(reg, BIN));

// with leading zeroes!

sprintf(buff, "The value of %d in (full) binary %s.", reg, fullBinary(reg));

Serial.println(buff);

// turn on (set) a specific bit (aka make it 1): bitwise OR (|)

uint8_t n = 5;

reg |= 1 << n;

sprintf(buff, "Setting bit %d yields %s.", n, fullBinary(reg));

Serial.println(buff);

// turn off (clear) a specific bit (aka make is 0): bitwise AND (&)

n = 2;

reg &= ~(1 << n);

sprintf(buff, "Clearing bit %d yields %s.", n, fullBinary(reg));

Serial.println(buff);

//Setting multiple bits in a Register

reg |= 1 << 7 | 1 << 4;

sprintf(buff, "Setting bits simultaneously yields %s.", fullBinary(reg));

Serial.println(buff);

//Clearing multiple bits in a Register

reg &= ~(1 << 5 | 1 << 4 | 1 << 1);

sprintf(buff, "Clearing bits simultaneously yields %s.", fullBinary(reg));

Serial.println(buff);

}

//Precondition: None

//PostCondition: The fully-formed 8-bit binary representation of the parameter is returned as a string

char* fullBinary(uint8_t value) {

static char result[9] = "";

for (uint8_t i = 0; i < 8; i++)

result[7 - i] = value & 1 << i ? '1' : '0';

return result;

}

void loop() {}

### Output