1.0 Minimal Confirmation Prototype (Trim Pot not used)

ATmega328 Datasheet: http://darcy.rsgc.on.ca/ACES/Datasheets/ATMEGA328P.pdf#page=111

1.1 ATmega328P Timers: 3 Options

1.2 ATmega328P Timer Presentation Pins (Timer1 Mapped to Digital Pins 9 and 10)

1.3 Timer1 (Fast PWM Mode 15): Servo Waveform Generation

1.4 Timer1 Waveform Generation Modes

1.5 Timer1 Mode 0 Waveform with Interrupt

1.5.1 Code Example: Register-Level Blink Sketch
// PROJECT  :Timer1NormalMode0Blink

// PURPOSE  :Simplest introduction to Register Level Coding

// COURSE   :ICS3U-E

// AUTHOR   :C. D'Arcy

// DATE     :2025 05 02

// MCU      :328P

// STATUS   :Working

void setup() {

//  pinMode(LED_BUILTIN, OUTPUT);

DDRB |= (1 << PB5);   // pinMode(13,OUTPUT);

// Register level...

cli();      //NoInterrupts();

// Normal Mode

TCCR1A = 0;

// set up timer with CLKio/256 prescaler

TCCR1B = 1<<CS12;

// Enable Timer1 interrupt ability

TIMSK1 = 1 << TOIE1;

// Enable global interrupt ability...

sei();      //Interrupts();

}

// Timer 1 Interrupt Service Routine

ISR(TIMER1_OVF_vect) {

PORTB ^= (1 << PB5);    //toggle state of pin 13

}

void loop() {}   //nothing to do...

1.6 ATmega328P Interrupt Vector Table: Column 2 Suitable for Direct Code Use

1.6.1 ISR Code Template

// Timer/Counter 1 Interrupt Service Routine

ISR(TIMER1_OVF_vect) {

PORTB ^= (1 << PB5);

}

1.7 Timer 1 Primary Control Registers and Prescalers

1.8 Timer 1 Secondary Registers

1.9 Timer1 Mode 14 Configuration Considerations

1.10.2 Register-Level PWM Waveform for Servo Rotation (ATmega328P Timer1 Mode 15)

// PROJECT  :ServoTimer1FSB103B2025

// PURPOSE  :Timer1 (Mode 15) Register Level Coding of an FSB10B Servo

// COURSE   :ICS3U-E

// AUTHOR   :C. D'Arcy

// DATE     :2025 05 02

// MCU      :328P (Standalone)

// PERIOD   :20ms > Frequency: 50Hz

// HIGH     :0Deg:1ms, 90Deg:1.5ms, 180Deg:2ms (Duty Cycle Range:5% to 10%)

uint8_t clkDiv1 = 1 << CS10;

uint8_t clkDiv8 = 1 << CS11;

uint8_t clkDiv64 = 1 << CS11 | 1 << CS10;

uint8_t clkDiv256 = 1 << CS12;

uint8_t clkDiv1024 = 1 << CS12 | 1 << CS10;

// The values of 0 and 38 arrived at from trial and error....

volatile uint8_t lowerLimit = 4;   //close to 0 degrees

volatile uint8_t upperLimit = 38;  //close to 180 degrees

volatile uint8_t hornSpeed = 4;    //# of Overflows reached before changing angle (duty cycle) in OCR1B

volatile uint8_t ovfCount = 0;

int main() {

cli();          // Suspend interrupts while we set up ours

// Configure for Fast PWM on OC1B (Arduino Pin 10)

// OC1A/OC1B: Set on Compare Match; Clear on BOTTOM

// Define WGM bits for Fast PWM (Mode 15)

TCCR1A |= (1 << COM1A1) | (1 << COM1B1) | (1 << WGM11) | (1 << WGM10);

// Set prescaler and additional WGM bits for Fast PWM

TCCR1B |= (1 << WGM13) | (1 << WGM12) | (1 << CS12) | (1 << CS10);

// Define TOP (See D'Arcy Excel worksheet)

// OCR1A = 328;

OCR1AH = 0x01;  // this is the same as the line

OCR1AL = 0x48;

// Define Duty Cycle for 0 degree horn angle...

// OCR1B = lowerLimit;

OCR1BH = 0;  // this is the same as the line

OCR1BL = lowerLimit;

// Enable Output Compare for Timer 1 interrupt

TIMSK1 = (1 << OCIE1A);

// Declare related peripheral pin(s) output

DDRB = (1 << PB2) | (1 << PB1);

// Enable Global Interrupt

sei();

// Stand down and admire your engineering

while (1);

}

// Timer1 Output Compare triggered on TCNT1 == TOP (OCR1A)

ISR(TIMER1_COMPA_vect) {

// adjust hornSpeed limit as required to affect speed or rotation...

ovfCount++;

if (ovfCount == hornSpeed) {

ovfCount = 0;

OCR1B++;

if (OCR1B == upperLimit)

OCR1B = lowerLimit;

}

}