// PROJECT  :RegisterLevelShiftOut

// PURPOSE  :Application of introduction to writing lower level code

// COURSE   :TEJ3M

// AUTHOR   :C. D'Arcy

// DATE     :2025 12 12

// MCU      :Nano

// STATUS   :Working

// http://darcy.rsgc.on.ca/ACES/images/BreadboardArduinoIntroPics063.png

// Any 3 pins will suffice but these have an advantage later in the course

#define DATA 1 << PB3   //  ATmega328p's Digital Pin 11 (MOSI)

#define CLOCK 1 << PB5  //  ATmega328p's Digital Pin 13 (SCK)

#define LATCH 1 << PB2  //  ATmega328p's Digital Pin 10 (SS)

void setup() {

DDRB |= DATA | CLOCK | LATCH;  //Set the 595's 3 Control lines to OUTPUT

}

//Register-Level Shiftout: Serial In, Parallel Out (SIPO)

void shiftout(uint8_t order, uint8_t value) {

PORTB &= ~LATCH;                               //pull the latch line low

uint8_t mask = order ? B10000000 : B00000001;  //set the mask

//Use masking to determine and shift out each of the 8 bits in 'value'

for (uint8_t i = 0; i < 8; i++) {

PORTB &= ~CLOCK;   //first, clear the clock (falling edge)

if (value & mask)  //identify the bit (either a 0 or a 1) to be shifted

PORTB |= DATA;   //set the bit

else               //or

PORTB &= ~DATA;  //clear the bit

PORTB |= CLOCK;    //set the clock bit (rising edge)

value = order ? value << 1 : value >> 1;  //shift all bits in ‘value’

}

//Done. Pull the latch line high to present 8 bits in parallel

PORTB |= LATCH;

}

//let's start with LSBFIRST (0)

uint8_t dir = LSBFIRST;

void loop() {

//Example 1

shiftout(dir, 0xF0);

while (1) {}

//Example 2

for (uint8_t i = 0; i < 8; i++) {

shiftout(dir, 1 << i);

delay(100);

}

dir = ~dir;

}