// PROJECT  :ShiftoutDACwithFeedback

// PURPOSE  :595 shift register feeding an 8-bit R/2R ladder

//           with ADC feedback

// COURSE   :TEJ3M

// AUTHOR   :C. D'Arcy

// DATE     :Created: 2025 10 17.

// MCU      :328p (Nano)

// STATUS   :Working

// REFERENCE:Using the Serial Plotter Tool:

//https://docs.arduino.cc/software/ide-v2/tutorials/ide-v2-serial-plotter/

#define DATA 11   //aka MOSI

#define CLOCK 8   //any pin will do (for now)

#define LATCH 10  //aka SS (Slave Select)

#define PACE 30   //speed of animation

#define DACOUT A5 //read the output of the DAC R/2R Ladder

#define VCC 5.0   //supply voltage

uint16_t count;   //ramp up the digital input to the DAC

uint16_t reading; //raw 10-bit ADC reading from the DAC

float voltage;    //map the raw ADC reading to the corresponding voltage

void setup() {

Serial.begin(9600);       //invoke the services of the Serial Plotter

while (!Serial) {}

pinMode(DATA, OUTPUT);    //prepare the three 595 inputs

pinMode(LATCH, OUTPUT);

pinMode(CLOCK, OUTPUT);

count = 0;                //start the count off at ground

}

void loop() {

digitalWrite(LATCH, LOW);               //disable output registers

shiftOut(DATA, CLOCK, MSBFIRST, count); //shiftout the 8-bits  
  digitalWrite(LATCH, HIGH);              //present to output pins

reading = analogRead(DACOUT);           //read DAC output(ADC)

voltage = reading*VCC/1024;             //convert to (fp) voltage

Serial.print("VCC:");                   //publish to the Serial Plotter

Serial.print(VCC);

Serial.print(',');

Serial.print("GND:");

Serial.print(0.0);

Serial.print(',');

Serial.print("ADC_Voltage:");

Serial.println(voltage);

delay(PACE);                            //pause...

count++;                                //continue, with rollover

}