### 1.5.1 Swapping Variables

Swapping the contents of two RAM locations, say x and y, in assembly, can be written as follows. (assume x is 5, y is 10, and temp is 15; [n] means the contents of RAM Address n)

| High Level | Machine Level |  | CHUMP (Assembly) Level | Comment |
| --- | --- | --- | --- | --- |
|  | Address | Instruction |  |  |
| temp = x | 0000 | 1000 0101 | READ x | addr←5, pc++ |
|  | 0001 | 0001 0000 | LOAD IT | accum←[5], pc++ |
|  | 0010 | 0110 1111 | STORETO temp | [15]←accum, pc++ |
| x = y; | 0011 | 1000 1010 | READ y | addr←10, pc++ |
|  | 0100 | 0001 0000 | LOAD IT | accum←[10], pc++ |
|  | 0101 | 0110 0101 | STORETO x | [5]←accum, pc++ |
| y = temp; | 0110 | 1000 1111 | READ temp | addr←15, pc++ |
|  | 0111 | 0001 0000 | LOAD IT | accum←[15], pc++ |
|  | 1000 | 0110 1010 | STORETO y | [10]←accum, pc++ |