// PROJECT  :SerialEchoAssembly.S

// PURPOSE  :AVR-GCC Assembly USART usage: echo characters to Serial Mon.

// RESULT   :Entered characters echoed on the Serial Monitor

// COURSE   :ICS4U-E

// AUTHOR   :C. D'Arcy

// DATE     :Created: 2025 02 09.  Confirmed: 2025 03 21.

// MCU      :328P

// STATUS   :Working.

// REFERENCE:Golam Mostafa (halfway down...)

// https://forum.arduino.cc/t/problems-in-serial-communication-with-registers/1016408/6

// P.10. https://raw.githubusercontent.com/abcminiuser/avr-tutorials/master/USART/Output/USART.pdf

// https://hekilledmywire.wordpress.com/2011/01/05/using-the-usartserial-tutorial-part-2/

// http://darcy.rsgc.on.ca/ACES/Datasheets/ATMEGA328P.pdf (Chapter 20)

.global main

#include "avr/io.h"

//The next two lines are the preferred strategy but I kept running into error: Missing ')' Bracket

//#define USART_BAUDRATE 9600        //Desired baud rate

//#define BAUD_PRESCALER (((F_CPU / (USART_BAUDRATE * 16UL))) - 1)

//Hard code the BAUD value to be loaded into the UBRRn register pair

.equ BAUD_PRESCALER, 103         ;precomputed from formula above

.equ data, 65                    ;ASCII value 65 translates to 'A'

.equ ASYNCHRONOUS, 0<<UMSEL01 | 0<<UMSEL00

.equ SYNCHRONOUS,  0<<UMSEL01 | 1<<UMSEL00

.equ PARITY_NONE,  0<<UPM01   | 0<<UPM00

.equ PARITY_EVEN,  1<<UPM01   | 0<<UPM00

.equ PARITY_ODD,   1<<UPM01   | 1<<UPM00

.equ CHAR_SIZE8,   0<<UCSZ02  | 1<<UCSZ01 | 1<<UCSZ00   ;8-bit

.equ STOP_BITS1,   0<<USBS0

.equ STOP_BITS2,   1<<USBS0

.org  0x0000       ;base address of Interrupt Vector (jump) Table (IVT)

reset:

rjmp   main

.org  0x0100       ;locate the start of the code well clear of the IVT

main:

rcall   init_USART      ;initialize baud, char size, parity, stop bits

ldi     r24,data        ;Sample: an ASCII character to be transmitted

rcall   TXCHAR          ;echo (transmit) it

rcall   RXCHAR          ;Monitor the input (receive) data....

rcall   TXCHAR          ;echo (transmit) it

rjmp    .-8             ;infinite loop: RXCHAR > TXCHAR > RXCHAR >...  ret

init_USART:

ldi   r16,hi8(BAUD_PRESCALER)       ;set the baud rate

sts   UBRR0H,r16                    ;"

ldi   r16,lo8(BAUD_PRESCALER)       ;"

sts   UBRR0L,r16                    ;"

ldi   r16,(1<<RXEN0) | (1<<TXEN0)   ;enable receive and transmit

sts   UCSR0B,r16                    ;"

ldi   r16, ASYNCHRONOUS | CHAR_SIZE8 | PARITY_NONE | STOP_BITS1

sts   UCSR0C,r16                    ;set Comm. Mode  Default: 8N1

ret

// Precondition:  Character data (1 byte) to be transmitted is in R24

// Postcondition: Contents of R24 are transmitted via the USART (should appear on the Serial Monitor)

TXCHAR:

lds   r16,UCSR0A          ;determine if the transmit buffer is empty

sbrs  r16,UDRE0           ;if the flag is set, exit the loop

rjmp  .-8                 ;loop back to the previous statement

sts   UDR0, r24           ;transmit data in r24

ret

// Precondition:  None

// Postcondition: Character received in the input buffer is stored in R24

RXCHAR:

lds   r16,UCSR0A          ;

sbrs  r16,RXC0            ;

rjmp  .-8                 ;

lds   r24,UDR0            ;

ret