# As an exercise, convert this Arduino C code to AVR Assembly

// PROJECT  :UARTPolled

// PURPOSE  :Serial UART: Register-Level C Code from mega328 Datasheet

// COURSE   :ICS4U-E

// AUTHOR   :Modified by C. D'Arcy for Arduino IDE

// DATE     :2025 02 27

// MCU      :328P

// STATUS   :Working!

// REF:http://darcy.rsgc.on.ca/ACES/Datasheets/ATMEGA328P.pdf#page=170

#define FOSC 16000000                 //clock(XTAL) Speed

#define BAUD 9600                     //desired baud rate

#define MYUBRR FOSC / 16 / BAUD - 1   //confirm formula in Datasheet

//int main() {    //Doesn't work: circumvents initializations in main.cpp

void setup() {

USART_Init(MYUBRR);                 //configure for polled communication

while (1)                           //forever...

USART_Transmit(USART_Receive());  //echo incoming serial data stream

}

void USART_Init(uint16_t ubrr) {

UBRR0H = ubrr >> 8;                 //set the baud rate

UBRR0L = ubrr;

UCSR0B = (1 << RXEN0) | (1 << TXEN0);     //enable receiver & transmitter

UCSR0C = 3 << UCSZ00;    	                 //8 data, No parity, 1 stop bit

}

char USART_Receive() {

while (!(UCSR0A & (1 << RXC0)));         //wait for data to be received

return UDR0;                             //return data from buffer

}

void USART_Transmit(char data) {

while (!(UCSR0A & (1 << UDRE0)));         //wait for empty Tx buffer

UDR0 = data;                              //placing data in buffer sends!

}

void loop() {}                              //nothing to do...

# Important USART Registers