Consider the following sketch that sweeps your FS5103B Servo motor under Timer1 Fast PWM Mode 15.

// PROJECT  :ServoTimer1FSB103B2025

// PURPOSE  :Timer1 Fast PWM (Mode 15) Register Level Coding of an FSB10B Servo

// COURSE   :ICS4U-E

// AUTHOR   :C. D'Arcy

// DATE     :Created: 2025 05 02. Updated: 2025 12 15.

// MCU      :328P (Nano)

// STATUS   :Working

// REFERENCE:https://www.easycalculation.com/engineering/electrical/avr-timer-calculator.php

//          :Excel Sheet:http://darcy.rsgc.on.ca/ACES/TEI4M/FastPWMOnTimer1.xlsx

// PERIOD   :20ms > Frequency: 50Hz

// HIGH     :0Deg:1ms, 90Deg:1.5ms, 180Deg:2ms (Duty Cycle Range:5% to 10%)

// The values of 0 and 38 resulted from trial and error....

volatile uint8_t lowerLimit = 4;   //close to 0 degrees

volatile uint8_t upperLimit = 38;  //close to 180 degrees

volatile uint8_t hornSpeed = 4;    //overflow threshold before updating horn

volatile uint8_t ovfCount = 0;     //

int main() {

cli();              // aka noInterrupts(); Suspend interrupts

// Configure for Fast PWM on OC1B (Pin 10)

// OC1B: Clear on Compare Match; Set on BOTTOM (OCR1A)

// Define WGM bits for Fast PWM (Mode 15)

TCCR1A |= (1 << COM1B1) | (1 << WGM11) | (1 << WGM10);

// Set prescaler and additional WGM bits for Fast PWM

TCCR1B |= (1 << WGM13) | (1 << WGM12) | (1 << CS12) | (1 << CS10);

OCR1A = 313;            // Define TOP (See Excel worksheet above)

// OCR1AH = 0x01;       // this is equivalent to the line above

// OCR1AL = 0x39;       // recommended to assign High byte first

OCR1B = lowerLimit;     // Define Duty Cycle for 0 degree horn angle...

//OCR1BH = 0;           // these statements achieve the same as the line above

//OCR1BL = lowerLimit;  // recommended to assign High byte first

TIMSK1 = (1 << OCIE1A); // Enable Output Compare for Timer 1 interrupt

DDRB |= 1 << PB2;       // Declare OC1B (Pin 10) as output

sei();                  // aka interrupts(); Enable Global Interrupt

while (1) {};           // Stand down and admire your engineering

}

// Timer1 Compare Match A triggered on TCNT1 == OCR1A (TOP)

ISR(TIMER1_COMPA_vect) {

// adjust hornSpeed limit as required to affect rotation speed...

ovfCount++;

if (ovfCount == hornSpeed) {

ovfCount = 0;

OCR1B++;

if (OCR1B == upperLimit)

OCR1B = lowerLimit;

}

}